## General

| Frequency coverage (Unit: MHz) | Receiver | 0.030–199.999* 400.000–470.000* |  |
| --- | --- | --- | --- |
|  | Transmitter | 1.810–1.999 3.500–3.800 7.000–7.200 10.100–10.150 14.000–14.350 18.068–18.168 21.000–21.450 24.890–24.990 28.000–29.700 50.000–52.000 144.000–146.000 430.000–440.000 |  |
| Mode |  | USB, LSB, CW, RTTY, AM, DV, FM, WFM (Rx only) |  |
| No. of memory channels |  | Memory channels: 500 channels (100 groups) Scan edges: 25 channels Call channels: 4 channels (2 channels x 2 bands) |  |
| Antenna connector |  | BNC connector (50Ω) (One connector for all bands) |  |
| Power supply requirement |  | 13.8 V DC ±15% (with external power supply) 7.4 V DC (with BP-272) |  |
| Operating temp. range |  | –10°C to +60°C; +14°F to +140°F |  |
| Frequency stability |  | Less than ± 0.5 ppm (–10°C to +60°C; 14°F to 140°F) |  |
| Frequency resolution |  | 1 Hz |  |
| Current drain |  | 13.8 V DC | 7.4 V DC |
|  | TX (Max. power) | Less than 3 A (10 W) | Less than 2.5 A (5 W) |
|  | RX (Max. audio) | 0.5 A (typical) | 0.8 A (typical) |
|  | RX (Standby) | 0.3 A (typical) | 0.5 A (typical) |
| Dimensions |  | 200×83.5×82 mm (W×H×D, projections not included) |  |
| Weight |  | 1.1 kg (approximate; including BP-272) |  |

* Some frequency bands are not guaranteed.

## Transmitter

| Output power |  | 13.8 V DC | 7.4 V DC |
| --- | --- | --- | --- |
|  | SSB/CW/RTTY/FM/DV | 0.1 to 10 W | 0.1 to 5 W |
|  | AM | 0.025 to 2.5 W | 0.025 to 1.25 W |
| Modulation system |  | SSB : Digital PSN modulation AM : Digital low power modulation FM : Digital reactance modulation DV : Digital GMSK modulation |  |
| Spurious emissions |  | Less than −50 dB (HF) Less than −60 dB (50 MHz) Less than −60 dB (144/430 MHz) |  |
| Carrier suppression |  | More than 50 dB |  |
| Unwanted sideband |  | More than 50 dB |  |

## Receiver

| Receiver system | 0.030 to 24.999 MHz | RF Direct Sampling |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 25.000 MHz and above | Down Conversion IF Sampling |  |  |  |
| Intermediate frequencies |  | 25.000 and above：38.85 MHz±0.5 MHz |  |  |  |
| Sensitivity (HF: Preamp-1 ON, 50 MHz: Preamp-2 ON, 144/430 MHz: Preamp ON) |  | 0.500 to 1.799 MHz | 1.800 to 29.999 MHz | 50 MHz | 144/430 MHz |
|  | SSB/CW（10 dB S/N) | － | 0.20 μV | 0.15 μV | 0.11 μV |
|  | AM（10dB S/N）13.0 μV | 13.0 μV | 2.0 μV | 1.0 μV | 1.0 μV |
|  | FM（12 dB SINAD） | 0.5 μV (28.000 to 29.700 MHz) |  | 0.25 μV | 0.18 μV |
|  | DV（1% BER）（PN9） | 1.0 μV (28.000 to 29.700 MHz) |  | 0.63 μV | 0.35 μV |
| Sensitivity for RED version (HF: Preamp-1 ON, 50 MHz: Preamp-2 ON, 144/430 MHz: Preamp ON) |  | 1.8–2.999 MHz | 3.0–29.995 MHz | 50 MHz | 144/430 MHz |
|  | SSB (BW=2.4 kHz) | 10 dBμV emf | 0 dBμV emf | –6 dBμV emf | –6 dBμV emf |
|  | AM (4 kHz, 60% modulation) | 16 dBμV emf | 6 dBμV emf | 0 dBμV emf | 0 dBμV emf |
|  | FM (7 kHz, 60% modulation) | 0 dBμV emf (28.000 to 29.700 MHz) |  | –6 dBμV emf | –6 dBμV emf |
| Selectivity (Filter: SHARP) | SSB（BW=2.4 kHz） | More than 2.4 kHz/－6 dB Less than 3.4 kHz/－40 dB |  |  |  |
|  | CW（BW=500 Hz） | More than 500 Hz/－6 dB Less than 700 Hz/－40 dB |  |  |  |
|  | RTTY（BW=500 Hz） | More than 500 Hz/－6 dB Less than 800 Hz/－40 dB |  |  |  |
|  | AM（BW=6 kHz） | More than 6.0 kHz/－6 dB Less than 10 kHz/－40 dB |  |  |  |
|  | FM（BW=15 kHz） | More than 12.0 kHz/－6 dB Less than 22 kHz/－40 dB |  |  |  |
|  | DV（12.5 kHz spacing） | Less than －50 dB |  |  |  |
| Spurious and image rejection ratio (SSB/CW/AM/FM) |  | HF: More than 70 dB* (except for ADC aliasing) 50 MHz: More than 70 dB* 144 MHz: More than 65 dB 430 MHz: More than 54 dB * At Intermediate frequency in 25–30 MHz or 50–54 MHz: More than 50 dB |  |  |  |
| Audio output power | Internal SP | More than 530 mW (12 Ω load at 10% distortion) |  |  |  |
|  | External SP | More than 200 mW (8 Ω load at 10% distortion) |  |  |  |